1. Essential Features (“Shall”)

- 4x4x4 NeoPixel LEDs: Assemble and control 64 NeoPixel LEDs.

- Pre-determined Patterns: Implement multiple lighting patterns, such as:

  - Sequential wave

  - Random sparkle

  - Layer-by-layer transitions

- Power Supply: Operate from a +9V to +12V mains adapter.

- Ambient Light Intensity Control: Adjust LED brightness dynamically based on ambient light levels.

2. Recommended Features (“Should”)

- Multiple Operating Modes:

  - Toggle between modes using Grove-compatible buttons.

  - Modes could include color themes, motion patterns, and dynamic effects.

3. Hardware Setup (Arduino + Grove System)

- Grove NeoPixel LED Strip: Instead of individual LEDs, consider using Grove’s NeoPixel-compatible strips or modules for simplicity.

- Grove Light Sensor: Plug-and-play compatibility for ambient light sensing.

- Grove Button: For changing modes.

- Grove Base Shield: To connect all modules to your Arduino with minimal wiring.

- Arduino Board: Compatible with the Grove system, such as Arduino Uno or Nano.

4. Circuit Overview

- Power Supply:

  - Use a Grove power module or connect an external adapter (9-12V).

  - Use an onboard voltage regulator to step down to 5V for the NeoPixels.

- Connections:

  - Data pin from NeoPixels to Arduino (pin 6 recommended).

  - Light sensor to analog pin (e.g., A0).

  - Button to a digital pin (e.g., D2).

5. Example Code Snippet

Here’s a basic starting code using the Adafruit NeoPixel library and Grove components:

#include <Adafruit_NeoPixel.h>

// NeoPixel setup

#define LED_PIN 6

#define NUM_LEDS 64

Adafruit_NeoPixel strip = Adafruit_NeoPixel(NUM_LEDS, LED_PIN, NEO_GRB + NEO_KHZ800);

// Grove light sensor pin

#define LIGHT_SENSOR_PIN A0

// Grove button pin

#define BUTTON_PIN 2

int currentMode = 0; // To track modes

void setup() {

strip.begin();

strip.show(); // Initialize all pixels to 'off'

pinMode(LIGHT_SENSOR_PIN, INPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP); // Grove button

}

void loop() {

// Read ambient light level

int lightLevel = analogRead(LIGHT_SENSOR_PIN);

int brightness = map(lightLevel, 0, 1023, 0, 255); // Map to brightness

// Check button press to change mode

if (digitalRead(BUTTON_PIN) == LOW) {

delay(200); // Debounce delay

currentMode = (currentMode + 1) % 3; // Cycle through modes

}

// Display pattern based on mode

switch (currentMode) {

case 0:

wavePattern(brightness);

break;

case 1:

sparklePattern(brightness);

break;

case 2:

layerTransition(brightness);

break;

}

}

void wavePattern(int brightness) {

for (int i = 0; i < NUM_LEDS; i++) {

strip.setPixelColor(i, strip.Color(brightness, 0, 0)); // Red wave

strip.show();

delay(50);

strip.setPixelColor(i, 0); // Turn off

}

}

void sparklePattern(int brightness) {

for (int i = 0; i < NUM_LEDS; i++) {

int randomLED = random(0, NUM_LEDS);

strip.setPixelColor(randomLED, strip.Color(0, brightness, 0)); // Green sparkle

strip.show();

delay(100);

strip.setPixelColor(randomLED, 0); // Turn off

}

}

void layerTransition(int brightness) {

for (int layer = 0; layer < 4; layer++) {

for (int i = layer * 16; i < (layer + 1) * 16; i++) {

strip.setPixelColor(i, strip.Color(0, 0, brightness)); // Blue layer

}

strip.show();

delay(500);

for (int i = layer * 16; i < (layer + 1) * 16; i++) {

strip.setPixelColor(i, 0); // Turn off

}

}

}

6. Next Steps

- Cube Assembly: Decide on how we will physically assemble the LEDs (e.g., use a 3D frame or stacked PCB layers).

- Circuit Design: a visual circuit diagram?

- Additional Patterns: ideas coding more complex patterns?